# Certified TestStand Developer Examination

Examinee: 	Date:

Administrator:	Date:

Instructions:

If you did not receive this exam in a sealed envelope stamped “NI Certification,” DO NOT ACCEPT this exam. Return it to the proctor immediately. You will be provided with a replacement exam.

- Do not detach the binding staple of any section. If any part of the exam paper is missing or detached when returned to National Instruments, you will be deemed to have failed the exam.

- This examination may not be taken from the examination area or reproduced in any way. You may not keep any portion of this exam after you have completed it.

- Do not ask the proctor for help. If any part of the exam is not clear, you may make appropriate assumptions and document them in a text document.

- A computer with a standard installation of LabVIEW, TestStand, and the software libraries required for the exam is the only reference allowed for the examination. Externally developed code or third party tools are not allowed in the exam. You may use any design patterns, templates, and examples available in the development environment as a guide/resource for the application development.

- The application must be specifically developed for the exam submission.

- The required software packages for the application are provided to you installed on the local machine under the users\public\documents directory. You must maintain the folder hierarchy and use these components to develop your application. Solutions that do not use the hierarchy or the given components are not graded.

- Do not rename any of the Vis, libraries or any other provided material.

IMPORTANT:

- When you complete the exam, <Instructions on exam completion here>

NON-DISCLOSURE AGREEMENT AND TERMS OF USE FOR NATIONAL INSTRUMENTS EXAMS

- This exam is confidential and is protected by trade secret law. It is made available to you, the examinee, solely for the purpose of becoming certified in the technical area referenced in the title of this exam.

- You are expressly prohibited from disclosing, publishing, reproducing, or transmitting this exam, in whole or in part, in any form or by any means, verbal or written, electronic or mechanical, for any purpose, without the prior express written permission of National Instruments - Training & Certification.

- By beginning work on the exam, you are accepting the NDA statement and agree not to disclose the content of this CTD Exam.

Section I: General Requirements

The Certified TestStand Developer exam tests your ability to develop a TestStand Sequence.

The application must do the following:

- Function as specified in Section II of this document

- Conform to TestStand style and documentation standards (Guide to Developing Effective, Maintainable TestStand Sequences)

- Be created expressly for this exam using step types and functions available in TestStand, along with code modules provided. Templates, examples, or code developed outside the bounds of this exam are not acceptable for use in the application

- Close all opened references and handles where used

- Be easily scalable to add more states/features without manually updating the hierarchy

- Pass Sequence Analyzer without errors.

Grading:

The CTD exam consists of a total of 100 points which are allocated as below. Exam passing grade is 70%

- Functionality (100 points)

Time allocated for the exam: 4 hours

# Objective

Design an automated sequence using NI TestStand to test a SP25 Circuit Card as per the specifications detailed below.  You must use the provided APIs and code to develop your application.

## SP25 Overview

The SP25 is a Maximum Power Point Tracking (MPPT) Charge Controller. It is designed for use in solar panel systems where the solar panels feed power to the SP25, which then adapts the power in order to charge batteries. The diagram below shows the SP25 as it would be used in a complete system.

Solar

Panels

SP25

Charge Controller

B00

A

Battery

Figure 1- SP25 shown in complete system view.

- Power input from the Solar Panels to the SP25. This is the only source of power to the SP25.

- Power output from the SP25 used to charge batteries.

## SP25 Operation

The SP25 is powered on when anything above 12V is applied on input A. After receiving power, it takes the SP25 3+ seconds to become operational.  Once operational the SP25 will begin to deliver power on output B.

The SP25 also includes an interface for configuring the chip for customers. A PC can be hooked up to it using USB, and through software shipped by the supplier: set and read certain configurations. The specific configurations of the controller are as follows

- Read/Write to set the output for 12V, or 24V mode.  This changes line B on Figure 1.

- Read/Write the fan to turn on at a certain temperature.

- Read the status of the SP25.

## SP25 Automated Testing Hardware Overview

The SP25 within the test configuration looks like this:

D

DMM

SP25

Charge Controller

C

A

Battery

DC Power Supply

B

PC

- Input lines from the DCPowerSupply to the SP25: used to imitate solar panels.

- Input/output lines from the PC to the SP25: used to communicate with the device.

- Output lines to a battery, also connected to the DMM. These lines are used for voltage/current reading.

- Output lines from the SP25 to the DMM. These lines are used for temperature, and fan information.

The SP25 is powered on when anything above 12V is applied on input A. After receiving power, it takes the SP25 at least 3 seconds for the output to relate to the input.  Once operational the SP25 will begin to deliver output on the lines C and D. While the device is connected to the PC, even if power is off, communication can happen.

DMM:

There are 9 active lines on the DMM. All of them are DC lines. These lines are only “active” for 10 seconds after you receive a “Ready” from the SP25 Status. After the 10 seconds, the lines will only report 0’s for voltages and currents.

The SP25 provides 8 status LED’s. The DC lines (0-7) of the DMM are connected to these 8 LED’s. Lines 0-6 are connected to the LED’s that correspond to the current temperature status of the SP25 charge controller. Line 7 is connected to the LED that indicates the current fan status (ON/OFF) of the SP25 charge controller.

Corresponding Temperature Lines (0-6) are represented in the table below

| Line 0 | Line 1 | Line 2 | Line 3 | Line 4 | Line 5 | Line 6 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 – 9.99 C | 10 – 19.99 C | 20 – 29.99 C | 30 – 39.99 C | 40 – 49.99 C | 50 – 59.99 C | 60 – 69.99 C |

The current temperature of the SP25 is represented by the respective LED being in the ON state (Digital Voltage >= 1V). Only one line shall be in the ON state. The remaining lines shall be in the OFF state (0V).

When the Fan is in the ON state, the DC Voltage of Line 7 shall be 5V (± 0.1V), and the DC Current shall be 0.4A (± 0.05A). When the Fan is in the OFF state, the DC Voltage shall be 0V and the DC Current shall be 0A.

Line 8 of the DMM is connected to the output from the SP25 to the battery. When the SP25 returns a status of “Ready” the output to the battery is enabled. The DC Voltage and Current shall be determined by the SP25 Mode Setting and the DC Power Supply setting; the Test Specifications below shall provide the corresponding settings and the expected outputs.

## VM1000 Test Specification

Using the APIs (found in Appendix A, B, and C) for the SP25 and instruments: Create an automated test to verify that the SP25 Charge Controller is fully functional and is sending/receiving signals properly. This will consist of ONE sequence file that contains all your code. Follow the bullets below to test at certain configurations and TestStand settings.

- TestStand Settings

  - Allow Tracing into Setup/Cleanup.

  - Allow Tracing While Terminating.

  - Tracing speed set to slowest possible.

  - Run-Time Error should show a Dialog Box.

  - Allow Prompt to Find Files.

  - Add a search directory for the location of the code modules “C:\Users\Public\Documents\CTD_Test”. This should search the subdirectories.

- Instrument Setup

  - Initialize the DCPowerSupply and set the current limit to 15A.

  - Initialize the DMM

  - Initialize the SP25 connection.

  - Ensure that the SP25 reports “OFF” as the status.

- Test 1

  - Turn off the output of the DCPowerSupply

  - Set the Fan to operate at 50-59.99C.

  - Read the fan to check if it got set correctly.

  - Set the mode to 12V.

  - Read the mode to check if it got set to 12V correctly.

  - Set the Voltage on the DCPowerSupply to 15V.

  - Turn on the output of the DCPowerSupply

  - Wait until the Status says “Ready”

  - Check that the temperature is 30-39.99C.

  - Check that the fan is off, check voltage and current.

  - Check that the output to the battery is 14V (+- 0.3), 2A (+- 0.05).

- Test 2

  - Turn off the output of the DCPowerSupply.

  - Set the Fan to operates at 30-39.99C.

  - Read the Fan to check if it got set correctly.

  - Set the mode to 12V.

  - Read the mode to check if it got set to 12V correctly.

  - Set the Voltage on the DCPowerSupply to 20V.

  - Turn on the output of the DCPowerSupply

  - Wait until the Status says “Ready”

  - Check that the temperature is 30-39.99C.

  - Check that the fan is on, check voltage and current.

  - Check that the output to the battery is 14V (+- 0.3), 4A (+- 0.05).

- Test 3

  - Turn off the output of the DCPowerSupply.

  - Set the Fan to operate at 40- 49.99C.

  - Read the Fan to check if it got set correctly.

  - Set the mode to 24V.

  - Read the mode to check if it got set to 24V correctly.

  - Set the Voltage on the DCPowerSupply to 30V.

  - Turn on the output of the DCPowerSupply

  - Wait until the Status says “Ready”

  - Check that the temperature is 40-49.99C.

  - Check that the fan is on, check voltage and current.

  - Check that the output to the battery is 27V (+- 0.3), 3A (+- 0.05).

- If the test fixture cannot be initialized

  - Suppress all errors (but mark the test as an error)

  - Record that it could not initialize the test fixture using an Additional Results step.

## VM1000 Automated Testing Software Overview

All APIs are supplied to you and can be found in C:\Users\Public\Documents. The following Appendix additions explain each function that can be used with each instrument. At the end of each table of explanations is a table of error codes and what they mean.

## Appendix A: SP25 API

This API oversees the communication between the PC and the SP25 Circuit Card.

To initialize the connection, you must pass in the correct information on the Initialize.vi Name parameter. Name must be “SP25”. After this you just need to hold onto the reference number.

| Function Name | Parameter Name | Direction | Type | Purpose | Errors |
| --- | --- | --- | --- | --- | --- |
| Initialize | Name | In | String | The name of the instrument. Internally this creates a reference to the instrument. | 5001 |
|  | Reference Out | Out | U32 | Numerical reference to the instrument. |  |
|  | Success | Out | Boolean | Indicates whether initialization was successful or not. |  |
| Write Mode | Reference In | In | U32 | Numeric reference to the instrument. | 5001, 5002 |
|  | 24V Mode | In | Boolean | Indicates whether to turn on 24V Mode. False = Turn Off, True = Turn On. |  |
|  | Reference Out | Out | U32 | Numeric reference to the instrument. |  |
| Read Mode | Reference In | In | U32 | Numeric reference to the instrument. | 5001 |
|  | Reference Out | Out | U32 | Numeric reference to the instrument. |  |
|  | 24V Mode | Out | Boolean | Indicates whether the 24V Mode is on. True = On, False = Off. |  |
| Read Status | Reference In | In | U32 | Numeric reference to the instrument. | 5001 |
|  | Reference Out | Out | U32 | Numeric reference to the instrument. |  |
|  | Status | Out | String | Indicates the status of the SP25. Possible answers are “Ready”, “Preparing”, “Off” |  |
| Write Fan | Reference In | In | U32 | Numeric reference to the instrument. | 5001, 5002 |
|  | Fan | In | Enum | Indicates what temperature to turn the fan on at: 0-9.99C, 10-19.99, 20-29.99, 30-39.99, 40-49.99, 50-59.99, 60-69.99. |  |
|  | Reference Out | Out | U32 | Numeric reference to the instrument. |  |
| Read Fan | Reference In | In | U32 | Numeric reference to the instrument. | 5001 |
|  | Reference Out | Out | U32 | Numeric reference to the instrument. |  |
|  | Fan | Out | Enum | Indicates what temperature the fan will turn on at: 0-9.99C, 10-19.99, 20-29.99, 30-39.99, 40-49.99, 50-59.99, 60-69.99. |  |
| Close | Reference In | In | U32 | Numeric reference to the instrument. | 5001 |
|  | Success | Out | Boolean | Indicates whether close was successful or not. |  |

Error Codes and Explanations

| 5000 | Boot error, instrument is not initialized. |
| --- | --- |
| 5001 | For Initialize, if Name passed in does not correspond to the instrument. |
| 5002 | The reference provided is invalid. |
| 5003 | Trying to change configuration while input has power. |

## Appendix B: DMM API

To initialize the DMM you must pass in the correct information on the Intialize.vi Visa resource name in parameter. DeviceName must be “COM1”.

| Function Name | Parameter Name | Direction | Type | Purpose | Errors |
| --- | --- | --- | --- | --- | --- |
| Initialize | DeviceName | In | String | Used to tell the function what alias the power supply uses to establish a connection. | 5010, 5013 |
|  | Visa resource name out | Out | VISA | The VISA resource after opening the connection to the instrument. |  |
|  | Success | Out | Boolean | True if successfully connected |  |
| Get DC Voltage | Visa resource name in | In | VISA | The active VISA resource connection. | 5010, 5011 |
|  | Channels | In | Array of U8 | Channels to read. |  |
|  | Visa resource name out | Out | VISA | The active VISA resource connection. |  |
|  | Voltages (Volts) | Out | Array of Double | Voltages on the channels passed in |  |
| Get DC Current | Visa resource name in | In | VISA | The active VISA resource connection. | 5010, 5011 |
|  | Channels | In | Array of U8 | Channels to read. |  |
|  | Visa resource name out | Out | VISA | The active VISA resource connection. |  |
|  | Currents (Amps) | Out | Array of Double | Currents on the channels passed in. |  |
| Get AC Voltage | Visa resource name in | In | VISA | The active VISA resource connection. | 5010, 5012 |
|  | Channel | In | U8 | Channel to read the voltage on |  |
|  | Visa resource name out | Out | VISA | The active VISA resource connection. |  |
|  | Voltage (Volts) | Out | Double | Voltage of the channel |  |
| Get AC Current | Visa resource name in | In | VISA | The active VISA resource connection. | 5010, 5012 |
|  | Channel | In | U8 | Channel to read the current on |  |
|  | Visa resource name out | Out | VISA | The active VISA resource connection. |  |
|  | Current (Amps) | Out | Double | Current for the channel |  |
| Close | Visa resource name in | In | VISA | The active VISA resource connection. | 5010 |
|  | Success | Out | Boolean | Returns true if the DMM was shut down correctly. |  |

Error Codes and Explanations

| 5010 | Instrument is not initialized. |
| --- | --- |
| 5011 | The channel given is not a DC channel |
| 5012 | The channel given is not an AC channel |
| 5013 | When calling initialize, an instrument with that name does not exist |

## Appendix C: Power Supply API

To initialize the Power Supply you must pass in the correct information on the Intialize.vi Visa resource name in parameter. DeviceName must be “COM2”.

| Function Name | Parameter Name | Direction | Type | Purpose | Errors |
| --- | --- | --- | --- | --- | --- |
| Initialize | DeviceName | In | String | Used to tell the function what alias the power supply uses to establish a connection. | 5020, 5021 |
|  | Visa resource name out | Out | VISA | The VISA resource after opening the connection to the instrument. |  |
|  | Success | Out | Boolean | True if successfully connected |  |
| Set Current Limit | Visa resource name in | In | VISA | The active VISA resource connection. | 5020, 5023 |
|  | Current Limit (Amps) | In | Number | The limit at which the power supply will stop supplying current to the UUT. |  |
|  | Visa resource name out | Out | VISA | The active VISA resource connection. |  |
| Set Voltage | Visa resource name in | In | VISA | The active VISA resource connection. | 5020, 5022 |
|  | Voltage (Volts) | In | Double | The voltage for the DC Power Supply |  |
|  | Visa resource name out | Out | VISA | The active VISA resource connection. |  |
| Set Output State | Visa resource name in | In | VISA | The active VISA resource connection. | 5020 |
|  | State | In | Boolean | If True the DC Power Supply will turn the voltage on to the UUT. If false no current or voltage will be emitted from the DC Power Supply |  |
|  | Visa resource name out | Out | VISA | The active VISA resource connection. |  |
| Get Voltage And Current | Visa resource name in | In | VISA | The active VISA resource connection. | 5020 |
|  | Visa resource name out | Out | VISA | The active VISA resource connection. |  |
|  | Voltage (Volts) | Out | Double | Returns the voltage being output. |  |
|  | Current (Amps) | Out | Double | Returns the current being drawn |  |
| Get Current Limit | Visa resource name in | In | VISA | The active VISA resource connection. | 5020 |
|  | Visa resource name out | Out | VISA | The active VISA resource connection. |  |
|  | Current Limit (Amps) | Out | Double | Returns what the current limit is set to. |  |
| Close | Visa resource name in | In | VISA | The active VISA resource connection. | 5020 |
|  | Success | Out | Boolean | Returns True if the DC Power Supply was shut down correctly. |  |

Error Codes and Explanations

| 5020 | Instrument is not initialized. |
| --- | --- |
| 5021 | When calling initialize, an instrument with that name does not exist |
| 5022 | The user tries to set the voltage to some number out of bounds |
| 5023 | The user tries to set the current to some number out of bounds |